Bridge-AI Smart mushroom Project

Project Overview

The Bridge-AI project is a key component of JHUB Africa's Smart Mushroom Farming initiative, an IoT-enabled agricultural innovation that leverages AI and sensor technology to optimize mushroom cultivation within modified shipping containers. The project demonstrates JHUB Africa's commitment to climate-smart agriculture and digital transformation under Dr. Nderu's leadership.

Bridge-AI Website Technology Stack

- Backend Framework: Python Flask – chosen for its lightweight architecture, flexibility, and ease of deployment

- Data Storage: JSON – utilized for lightweight, human-readable data serialization

- Version Control: GitHub – enabling collaborative development and change tracking

- Deployment Platform: Render – cloud hosting for testing and production environments

Key Development Milestones

Week 1-2: Requirements Gathering (complete)

- Participated in stakeholder sessions with project manager and IoT engineers

- Documented user stories for smallholder farmers as primary users

- Defined functional and non-functional requirements for the Bridge-AI website

Week 3-4: Backend Development & Frontend Integration (complete)

- Set up Flask application with core routing and request handling

- Implemented JSON-based data models for project information and configurations

- Established GitHub repository with version control workflows

- Developed responsive user interfaces with authentication and dashboard functionality

Week 5-6: Website Testing & Deployment (complete)

- Conducted unit, integration, and user acceptance testing

- Optimized application performance and security measures

- Deployed on Render with environment configuration and dependency management

- Prepared deployment documentation and user guides

Week 7: Farmers' Smart Dashboard Planning(In progress)

- Began planning the dashboard data model and architecture

- Identified key entities: sensor data, user profiles, grow room configurations, alert settings

- Created initial design concepts for mobile-responsive UI

- Tech stack for dashboard and database to be determined based on project requirements.

Bridge-AI API Architecture

The system follows a REST API-first approach, allowing IoT devices to communicate with the Flask backend using HTTP requests and responses. This architecture supports:

- Real-time data processing and visualization

- Model versioning and tracking

- Scalability for future expansion

System Context Diagram

Key Achievements

- Successfully developed and deployed Bridge-AI website with Python Flask

- Implemented JSON-based data storage for project information

- Established GitHub version control for 16-member team collaboration

- Deployed on Render for stakeholder testing and demonstration

- Began planning farmers' smart dashboard with mobile-responsive design

IOT SECTOR

# Progress Summary

Following the research phase, the IoT subsystem was successfully simulated using Wokwi. The simulation validated the proposed control strategy by demonstrating automatic monitoring and regulation of temperature, humidity, lighting and other environmental conditions. The project has now progressed to the structural design phase, where the mushroom growing room is being modelled using SketchUp.

# Research and Requirements Analysis

Activities completed include:
• Literature review on mushroom farming techniques.
• Study of mushroom growth stages and environmental requirements.
• Selection of suitable sensors, actuators and controllers.
• Identification of suitable communication technologies.
• Investigation of energy-efficient room designs.
• Definition of the overall IoT architecture.

# Virtual Simulation Progress

The complete control system has been virtually simulated using the Wokwi simulation platform. The ESP32 microcontroller was selected as the primary controller responsible for collecting sensor data and controlling the environmental conditions.

The following modules were successfully simulated:
• Temperature and humidity sensing using the DHT22 sensor.
• LCD display for real-time environmental monitoring.
• Relay-controlled outputs representing the fan, heater, humidifier and lighting system.
• Environmental control logic based on mushroom fruiting-stage requirements.
• Automatic switching of actuators according to sensor readings.

The successful simulation confirms that the proposed control strategy is feasible before physical hardware implementation.

## Figure 1. ESP32-based Wokwi simulation showing environmental monitoring and control.

## SYSTEM ARCHITECTURE

The IoT subsystem operates within a 20-foot modified shipping container, equipped with sensors and actuators controlled by a central microcontroller (ESP32/Raspberry Pi). The system enables remote monitoring via a smartphone app, empowering smallholder farmers with precision agriculture capabilities.

Sensor Components

| Sensor | Purpose | Specifications |
| --- | --- | --- |
| DHT22 | Temperature & Humidity Monitoring | ±0.5°C accuracy, 0-100% RH range |
| MH-Z19 | CO₂ Concentration Monitoring | 0-5000 ppm, ±50 ppm accuracy |
| BH1750 | Light Intensity Detection | 1-65,535 lux range |
| Soil Moisture | Substrate Moisture Monitoring | Capacitive sensing |

Actuators

| Actuator | Purpose | Control Mechanism |
| --- | --- | --- |
| Exhaust Fans | Temperature & CO₂ Regulation | Activated at thresholds |
| Misting System | Humidity Regulation | Automated misting cycles |
| UV/LED Lights | Light Cycle Management | Programmable schedules |

Data Flow Architecture

Automated Microclimate Control

The system implements stage-aware climate control, adjusting environmental parameters based on mushroom growth phases:

- Temperature: Maintained at optimal range for each growth stage

- Humidity: Controlled between 90-100% for ideal conditions

- CO₂ Regulation: Automatic ventilation when thresholds exceeded

- Light Cycles: Programmable illumination schedules

Integration with Bridge-AI

- Sensor data streams to API via HTTPs

- Actuator commands processed through AI-driven decision engine

- Real-time alerts and notifications to farmers via smartphone

- Historical data stored for analytics and predictive modeling

# Structural Design Progress

The team is currently finalizing the structural design of the mushroom growing room using SketchUp. The proposed room measures approximately 6 m × 2.4 m × 2.4 m and is designed to provide a controlled indoor environment.

The design includes:
• Insulated walls and roof.
• Concrete floor.
• Four mushroom growing racks.
• Fresh-air intake and exhaust ventilation.
• Water storage tank.
• Control cabinet housing the ESP32, Raspberry Pi and relay modules.
• Dedicated locations for sensors and actuators.
• Central maintenance walkway.

The room layout has been planned to maximize airflow uniformity, simplify maintenance and reduce overall energy consumption

Caption: Figure 2. Proposed SketchUp mushroom growing room structure.

# Current Status

At this stage, the project has successfully completed:
• Research and requirement analysis.
• Selection of hardware components.
• IoT system architecture.
• Virtual simulation and validation.
• Preliminary room structural design.

The project is therefore ready to transition into physical prototype construction.

# Next Activities

The next phase will involve:
• Completing the SketchUp room model.
• Procuring project hardware.
• Building the physical mushroom room.
• Installing sensors and actuators.
• Developing the Raspberry Pi dashboard.
• Integrating machine learning for predictive environmental control.
• Testing, calibration and energy optimization.

# Conclusion

The Smart Mushroom Farming System has made significant progress. The research phase has established a strong technical foundation, while the successful virtual simulation demonstrates the feasibility of the proposed automation strategy. The ongoing structural design provides a practical layout for integrating the IoT components into a controlled mushroom cultivation environment. The project is on schedule to proceed to hardware implementation and system integration.