# BRIDGE AI SMART MUSHROOM SYSTEM

PROPOSED ROOOM STRUCTURE AND COMPONENTS

Room structure Dimensions

The room structure dimensions proposed are 3.5 m in length, 3m in width and 2.5 m height.

Component selection and placement.

HARDWARE COMPONENTS REQUIRED

1. SENSORS

A) TEMPETAURE AND HUMIDITY

DHT 22 SENSOR (3)

B) CO2 sensor

MH-Z19B / SCD 30 (2)

C) Water level SENSOR (1)

D) LIGHT SENSOR/LDR/ BH 1750 (1)

2. CAMERA MODULE/CCTV (1)

3. MICROCONTROLLERS

ESP 32 (1)

Raspberry Pi for edge computing. (1)

GSM module (1)

4. ACTUATORS

A) Humidity actuator

Fogger/ Ultrasonic Mist Maker (4)

B) CO2 Actuator

Exhaust/ Ventilation Fan 1(1)

C) Temperature Actuators

Heating element and Cooling Fan (2)

D) Substrate Actuator

Pump for water Regulation (1)

E) Light Actuator (LEDs)

5. BUZZER (1)

6. JUMPER WIRES/BREADBOARS/RESISTORS (220 OHMS AND 1k OHMS)/capacitors/inductors/SWITCHES/PCD BOARD/ETCHANT (HCL)/FEMALE AND MALE HEADER PINS/SOLDIERING WIRE AND GUN/ GLUE GUN

7. RELAY (1)

8.POWER SOURCE: SOLAR PANEL/LITHIUM BATTERY/ BUCK and BOOST CONVERTORS/VOLTAGE REGULATOR.

### SENSORS

- DHT 22(Temperature and humidity sensor):

- One DHT22 sensor works effectively within a space of approximately 28–56 m³.

- Our room has a volume which falls within this range.

- However, we decided to use 3 DHT22 sensors to improve measurement accuracy and provide redundancy in case one sensor fails.

Figure 1: DHT sensors are represented by the blue cubes

- MH-z19B / SCD 30 (CO2 sensors):

- MH-z19B has a measurement range of 0–5000 ppm and is cheaper than SCD 30

- SCD 30 has a measurement range of 0–40,000 ppm and higher accuracy.

- We preferred using SCD 30 as the CO2 sensor (WHY: During the incubation stage our CO2 levels are between 10,000ppm-20,000 ppm)

- Therefore, we require one SCD30 sensor.

- Positioning: Middle of the wall opposite the shelf racks.

Figure 2: SCD 30 is represented by the beige/ brown box on the right wall

- BH 1750 (Light Sensor):

- Has a measurement range of 1–65,535 lux.

- Only one BH1750 sensor is required.

- Positioning: Upper section of the wall opposite the shelf racks.

Figure 3: BH 1750 represented by the circular object with something poking out on the right wall

### ACTUATORS

- Exhaust fan

- One exhaust fan with over 100CFM

- Mushrooms need an active fresh air exchange rate of 4 to 8 full air changes per hour (ACH)

- Positioning: Back wall top left corner

Figure 4: Exhaust fan is represented by the fan at the top back wall

- Humidifier-Ultrasonic fogger

- For a grow room the output range of the humidifier should be between 500-3000ml/hour

- This can be achieved using a 3-head or 4-head ultrasonic fogger. (Note: Because it outputs 900 to 1,500 mL of water per hour, running it continuously will quickly turn your room into a dense, foggy swamp.)

- From figure 4, the humidifier is positioned on the ceiling

- Cooling fan

- Heater

- Positioning: Bottom of the wall opposite the shelves (WHY?: Cold air is denser and sinks placing the heater low means it's warming the coldest air first heat naturally rises and distributes upward through the room on its own)

- LED strips

- Because of the wide stacked shelf placing an LED at the ceiling may prevent light from reaching the lower shelves

- Solution: Install waterproof IP65 or IP67 LED strips along the underside of each shelf.

- 2W LED strips is sufficient.

Figure 5: LED strips are represented by the line underneath the shelf plus the yellow box above the top shelf